📡 ANTENNA SIGNAL LEAKAGE DETECTION SYSTEM Technical Documentation & Developer Reference

Built with

Python  ·  Streamlit  ·  Plotly  ·  NumPy

| Version 1.0.0 | Domain EM Security |
| --- | --- |

─────────────────────────────────────────

S⃗ = E⃗ × H⃗  |  G(θ) = cosⁿ(θ) + Σ side lobes  |  SLL = 20 log₁₀(Gₛᵇ/Gₘₐᵡ)

# 1. Application Overview

The Antenna Signal Leakage Detection System is a web-based simulation dashboard built with Python and Streamlit. It models the electromagnetic radiation pattern of a phased-array antenna, identifies side lobes where energy leaks unintentionally, quantifies security risks, and provides real-time interactive controls for engineers and researchers.

Core Physics Principle The Poynting vector S⃗ = E⃗ × H⃗ describes the directional energy flux density (W/m²) of an electromagnetic wave. This application uses it to visualize WHERE radiated power flows, and to detect angular directions where power leaks outside the intended main beam.

## 1.1  What the App Simulates

The application simulates three interrelated electromagnetic phenomena:

- Radiation Pattern G(θ): The directional gain of an antenna as a function of azimuth angle θ (0°–360°).

- Poynting Vector Field |S⃗|: A 2-D heatmap of energy flux intensity in the XY plane around the antenna.

- Side Lobe / Leakage Detection: Automatic identification of local gain maxima that exceed a configurable threshold and are angularly separated from the main beam.

## 1.2  Technology Stack

| Language | Python 3.9+ |
| --- | --- |
| Web Framework | Streamlit — dashboard, widgets, layout |
| Plotting Library | Plotly — all interactive charts and plots |
| Numerics | NumPy — array math, FFT, integration |
| Data | Pandas — export and comparison tables |
| Export | JSON / CSV download buttons (built-in Streamlit) |
| Launch Command | streamlit run app.py |

# 2. Code Architecture

The application is structured as a single file (app.py) divided into clearly separated logical layers:

| Layer | Responsibility |
| --- | --- |
| PAGE CONFIG & CSS | Streamlit page setup, custom dark-theme CSS injected via st.markdown() |
| SIMULATION ENGINE | 4 pure functions: generate pattern, detect leakage, compute metrics, spy analysis |
| PLOT BUILDERS | 6 Plotly figure factory functions (polar, heatmap, bar, gauge, animation) |
| EXPORT HELPERS | Serialize results to JSON and CSV for download |
| SIDEBAR (build_sidebar) | All user controls — returns a params dict consumed by main() |
| MAIN ORCHESTRATOR | Calls all layers in order; renders layout, alerts, cards, charts, tables |

Data flow: Sidebar controls → params dict → Simulation Engine → metrics + leakage list → Plot Builders → st.plotly_chart(). All computation is stateless and re-runs on every widget interaction.

# 3. Simulation Engine — Function Reference

The simulation engine is the mathematical core of the application. It consists of four pure Python functions that have no Streamlit dependencies and can be unit-tested independently.

## 3.1  generate_radiation_pattern()

| generate_radiation_pattern() Computes antenna gain G(θ) for every angle in the input array |  |
| --- | --- |
| Returns | np.ndarray of gain values (linear scale, same length as theta) |
| theta | np.ndarray — angles in radians from 0 to 2π |
| main_lobe_sharpness | int — exponent n in cosⁿ(θ); higher = narrower beam |
| side_lobe_strength | float 0–1 — relative amplitude multiplier for side lobes |
| num_side_lobes | int — how many side lobe harmonics to generate |
| noise_level | float — std-dev of Gaussian noise added to the pattern |
| steering_angle | float (degrees) — offsets the entire pattern by this angle |
| optimized | bool — if True, multiplies side_lobe_strength by 0.12 (Chebyshev taper) |

Mathematical model:

- Main lobe: Gₘₐᵡ(θ) = max(0,  cos(θ − θ₀))ⁿ  — a raised-cosine beam steered to θ₀

- Side lobes: Σ Aₖ · e^⁻⁰ʸ³ᵏ · |cos(θ ± k·Δφ − θ₀)|²  where Δφ = 2π / (2N) and amplitude decays with index k

- Noise: |N(0, σ)| added element-wise; absolute value keeps gain non-negative

- Optimized mode: Scales side_lobe_strength × 0.12, simulating a −30 dB Chebyshev window taper

## 3.2  detect_leakage_zones()

| detect_leakage_zones() Finds all side lobe peaks that exceed the security threshold |  |
| --- | --- |
| Returns | list[dict] — sorted by gain descending |
| theta | np.ndarray — angle array (radians) |
| gain | np.ndarray — gain array from generate_radiation_pattern() |
| threshold | float — minimum gain to consider a leakage (linear) |
| min_angular_sep | float (degrees) — angular distance from main lobe to ignore |

Each returned dict contains:

- angle_deg — direction of the leakage peak in degrees (0–360)

- gain — linear gain value at that peak

- gain_dB — Side Lobe Level in dB relative to the main lobe: 20·log₁₀(Gₛᵇ/Gₘₐᵡ)

- risk — "🔴 HIGH" if SLL > −10 dB, "🟠 MEDIUM" if > −20 dB, "🟢 LOW" otherwise

Detection algorithm: For each sample point, the function checks (1) gain exceeds threshold, (2) it is a local maximum relative to its two neighbors with wrap-around, and (3) it is at least min_angular_sep degrees away from the global maximum. This ensures the main lobe is never flagged as a leakage zone.

## 3.3  compute_metrics()

| compute_metrics() Derives all quantitative energy and security metrics from the pattern |  |
| --- | --- |
| Returns | dict with 11 keys |
| main_gain_dB | Peak gain of the main lobe in dBi |
| total_energy | np.trapz(gain, theta) — total radiated energy (a.u.) |
| main_energy | Energy integrated over the −3 dB main beam region |
| leaked_energy | Sum of gains at all detected leakage peak positions |
| efficiency | main_energy / total_energy × 100 (%) |
| risk_score | 0–100 composite score: 60% SLL weight + 40% leakage count weight |
| risk_label | "LOW" / "MEDIUM" / "HIGH" based on risk_score thresholds |
| hpbw | Half-Power Beamwidth: angular width of the −3 dB region |
| main_angle_deg | Direction of peak gain (steering angle verification) |

Risk score formula:

risk_score = round( (0.6 × SLL_weight + 0.4 × count_weight) × 100 )

- SLL_weight = clamp((max_SLL_dB + 40) / 40, 0, 1)  — maps [−40 dB, 0 dB] → [0, 1]

- count_weight = clamp(N_leakage / 10, 0, 1)  — 10 or more leakage zones = max weight

## 3.4  spy_antenna_analysis()

| spy_antenna_analysis() Evaluates interception risk from a passive eavesdropper at a given angle |  |
| --- | --- |
| Returns | dict with 5 keys |
| spy_angle | Input angle in degrees |
| spy_gain | Gain value at the spy position (linear) |
| sll_dB | Signal level at spy relative to main lobe (dB) |
| pct_main | spy_gain / main_gain × 100 (%) |
| intercepted | True if spy_gain > 5% of main lobe power (≈ −26 dB threshold) |

# 4. Visualization Layer — Plot Functions

## 4.1  polar_plot()  — Main Radiation Pattern

The central visualization. Renders G(θ) in dB on a polar axis (range −45 to +3 dB). Contains up to 5 overlaid traces:

- G(θ) pattern (blue): The normalized gain curve with a semi-transparent fill showing the beam's angular coverage.

- Threshold ring (amber dashed): A constant-radius circle at the configured leakage threshold, helping engineers see which parts of the pattern exceed the security limit.

- Main lobe star (yellow ★): Marks the peak gain direction. Clicking opens a Streamlit prompt for deeper analysis.

- Leakage markers (red ●): Scatter points at every detected side lobe peak. Hover shows angle, dB level, and risk classification.

- Spy antenna (triangle): Red triangle-up if intercepting, green triangle-down if safe. Position controlled by the sidebar slider.

- Optimized overlay (green, comparison mode): A second gain curve showing the pattern after Chebyshev tapering, enabling visual before/after comparison.

## 4.2  poynting_heatmap()  — |S⃗| Field Map

Renders a 200×200 pixel heatmap of the Poynting vector magnitude |S⃗| ∝ G(θ)/r in the Cartesian XY plane. For each grid point (x, y): the angle θ = atan2(y, x) is computed, G(θ) is interpolated from the 1-D pattern, and |S| is scaled by 1/r to approximate near-field decay. Color scale goes from deep navy (low intensity) through blue and amber to red (high intensity).

## 4.3  energy_bar_chart()  — Angular Energy Distribution

Divides the full 360° into 36 sectors of 10° each, integrates the gain curve within each sector using np.trapz(), and renders a bar chart. Bar color maps from cool blue (low energy) to warm red (high energy), giving an at-a-glance view of energy concentration.

## 4.4  sll_gauge()  — Risk Score Gauge

A Plotly Indicator gauge showing the 0–100 risk score. Three colored zones in the gauge background: green (0–30), amber (30–65), red (65–100). The bar and number color dynamically match the current risk level. The delta indicator compares to the neutral midpoint of 50.

## 4.5  time_simulation_chart()  — Animated Pattern

Generates 40 animation frames. Each frame adds a sinusoidal jitter to the gain pattern: Gₙ(t) = G(θ) + 0.05·sin(3θ + 2πt/40), simulating mechanical vibration or electronic beam scanning. Play/Pause buttons are rendered below the chart.

# 5. Sidebar Controls & UI Layout

## 5.1  Sidebar Control Reference

All sidebar controls are defined in build_sidebar() and returned as a single params dict. The main() function consumes this dict for all downstream computation.

| Control | Range / Type | Default | Effect on Simulation |
| --- | --- | --- | --- |
| Main Lobe Sharpness | 1 – 30 | 8 | Exponent in cosⁿ(θ); 1 = omnidirectional, 30 = near-pencil beam |
| Side Lobe Strength | 0.0 – 1.0 | 0.35 | Amplitude multiplier for all side lobe harmonics |
| Number of Side Lobes | 1 – 10 | 4 | How many harmonic components generate side lobes |
| Noise / Randomness | 0.0 – 0.4 | 0.05 | Std-dev of ¦N(0,σ)¦ added to the gain pattern |
| Beam Steering θ₀ | −90° to +90° | 0° | Shifts the entire pattern by this angle offset |
| Leakage Threshold % | 1 – 50 % | 15 % | Minimum gain (% of peak) to flag a point as leakage |
| Min Angular Sep. | 5° – 60° | 20° | Minimum angular distance from main lobe for a leakage flag |
| Spy Angle | 0° – 359° | 45° | Direction where the virtual spy antenna is placed |
| Comparison Mode | Toggle | OFF | Overlays optimized (Chebyshev tapered) pattern for comparison |
| Poynting Vector Map | Toggle | ON | Renders the 2-D ¦S⃗¦ heatmap |
| Energy Bar Chart | Toggle | ON | Renders angular energy distribution bar chart |
| Animated Simulation | Toggle | OFF | Enables the animated time-domain pattern |
| Angular Resolution | 360/720/1440/2880 | 720 | Number of sample points around the 360° circle |

## 5.2  Dashboard Layout (Top to Bottom)

- Alert Banner: One of three Streamlit alerts (st.success / st.warning / st.error) based on risk_label. A second alert appears if the spy antenna is actively intercepting.

- 6 Metric Cards: Risk Score, Efficiency %, Leakage Count, HPBW, Main Gain (dBi), Main Angle. Rendered as custom HTML divs with a dark card style.

- Polar Plot + Gauge: 3:1 column split. Polar chart on the left occupies 75% of width; the risk gauge and quick-stats table fill the right 25%.

- Poynting Map + Energy Bar: 50/50 split. Both togglable independently from the sidebar.

- Animated Simulation: Full-width animated polar chart with Play/Pause controls (only shown when toggled on).

- Leakage Zone Table: HTML table listing every detected side lobe with angle, linear gain, SLL in dB, and color-coded risk badge. Row background color reflects risk level.

- Deep Analysis (conditional): Appears only when Comparison Mode or spy interception is active. Shows before/after optimization table and detailed spy metrics.

- Hardening Recommendations: 6 engineering countermeasures rendered as a 3-column card grid.

- Export Section: Three download buttons: full JSON report, leakage zones CSV, complete radiation pattern CSV.

# 6. Physical & Mathematical Background

## 6.1  Poynting Vector

The Poynting vector S⃗ = E⃗ × H⃗ (W/m²) is the instantaneous power flux density of an EM wave. Its time average is ⟨S⃗⟩ = ½ Re(E⃗ × H⃗*). In the far field of a linear antenna:

- Eθ = jη · (I₀L / 2λr) · sinθ · e^⁻ʲᵏʳ

- Hφ = Eθ / η   where η = 120π Ω (free-space impedance)

- ⟨Sᵣ⟩ = |Eθ|² / 2η  ∝ G(θ) / r²

The 1/r² decay with distance is the fundamental reason why side lobes, even at −13 dB below the main lobe, remain interceptable at practical distances.

## 6.2  Side Lobe Level (SLL) and Security Implications

| SLL = 0 dB | Side lobe equals main lobe — catastrophic leakage |
| --- | --- |
| SLL = ∓6 dB | 50% of main lobe power — HIGH risk (25% intercept range of main) |
| SLL = −13.3 dB | First side lobe of uniform array — HIGH / MEDIUM boundary |
| SLL = −20 dB | 10% of main lobe power — MEDIUM / LOW boundary in this app |
| SLL = −30 dB | Chebyshev-tapered array — LOW risk for most threat models |
| SLL = −40 dB | Highly optimized array — negligible leakage |

## 6.3  Countermeasures Implemented in the App

- Chebyshev Taper (Optimized Mode): Multiplies side_lobe_strength by 0.12. In a real system, this corresponds to applying non-uniform excitation weights across array elements to achieve a guaranteed SLL of −30 dB at the cost of a ~15% wider main beam.

- Beam Steering: The θ₀ parameter shifts all lobes. Engineers use this to steer nulls toward known eavesdropper positions.

- Threshold-Based Detection: The configurable threshold allows operators to trade off between sensitivity (catch more weak leakage) and specificity (avoid false alarms from noise bumps).

# 7. Installation, Running & Extending

## 7.1  Requirements

- Python 3.9 or later

- pip install streamlit plotly numpy pandas

- No additional configuration required

## 7.2  Run the Application

Launch Command streamlit run app.py The browser opens automatically at http://localhost:8501

## 7.3  Common Extension Points

- Real antenna data: Replace generate_radiation_pattern() with a function that reads measured gain data (CSV) and normalizes it. All downstream functions are data-agnostic.

- 3-D pattern: Add a second elevation angle φ and extend the polar_plot() function to use Plotly's Scatterpolar3d or Surface trace.

- FHSS simulation: Call generate_radiation_pattern() in a loop with varying frequency (changes k = 2πf/c and therefore d/λ ratio), then animate across frequency steps.

- Multiple antennas: Sum array factors from multiple generate_radiation_pattern() calls with different steering angles to simulate MIMO or diversity systems.

- Live SDR data: Replace the gain computation with real-time samples from a software-defined radio (e.g., RTL-SDR via pyrtlsdr), feeding measured |S| values into the heatmap.

## 7.4  Export Files

| JSON Report | Full simulation state: all parameters, all metrics, all leakage zones with risk classifications. Timestamp included. Suitable for audit logs. |
| --- | --- |
| Leakage Zones CSV | Tabular list of detected side lobes: angle_deg, gain, gain_dB, risk. Suitable for further analysis in Excel or Python. |
| Radiation Pattern CSV | Complete gain curve: angle_deg, gain_linear, gain_normalized, gain_dB for all N sample points. Suitable for importing into MATLAB or antenna simulation tools. |

📡 Antenna Signal Leakage Detection System S⃗ = E⃗ × H⃗ ¦ G(θ) = cosⁿ(θ-θ₀) + Σ Aₖ e^⁻αᵏ ¦cos(θ±kΔφ-θ₀)¦²